ABSTRAK PEMANFAATAN LIMBAH CANGKANG TELURDANSEKAM PADI SEBAGAI KATALIS DALAM PEMBUATANBIODIESELDARI MINYAK JELANTAH

(Aulia Syafitri, 2022, 41 halaman, 12 Tabel, 9 Gambar, 4 Lampiran)

Sintesa biodiesel selama ini lebih banyak menggunakan katalis homogen berupaNaOH atau KOH. Penggunaan katalis homogen NaOH atau KOHdalampembuatan biodiesel memiliki beberapa kelemahan, diantaranya terbentuknyaproduk samping berupa sabun, dan rumitnya pemisahan produk biodiesel yangdihasilkan dengan katalis. Untuk mengatasi kelemahan tersebut, mulai dikembangkan penggunaan katalis heterogen (padat) untuk menggantikan katalishomogen (basa). Katalis homogen dapat digantikan dengan katalis heterogenyanglebih ramah lingkungan, stabil pada suhu tinggi, memiliki pori yang lebih besar dan harga relatif murah. Pembuatan katalis heterogen dapat dilakukan denganpemanfaatan limbah sekam padi dan cangkang telur yang menghasilkan katalisCaO/SiO2. CaO yang digunakan berasal dari limbah cangkang telur ayam. Komponen utama dari cangkang telur yakni CaCO3 dapat diubah menjadi CaOmelalui proses kalsinasi. Pemanfaatan sekam padi akan menghasilkan silika yangdapat dimanfaatkan untuk berbagai proses kimia, diantaranya adalah sebagai support katalis. Penelitian ini memvariasikan rasio CaO/SiO2 (5:3, 5:5, dan5,7) b/b serta waktu impregnasi (22, 23, dan 24) jam. Katalis terbaik dilihat dari yielddan mutu biodiesel yang dihasilkan. Hasil yield terbaik yang didapatkan adalah71,54% pada rasio 5:7 dengan waktu impregnasi 24 jam.

Kata Kunci: Katalis, Cangkang Telur, Sekam Padi, CaO/SiO2,Biodiesel

ABSTRACT UTILIZATION OF EGGSHELL WASTE AND RICE HUSKAS A CATALYST IN BIODIESEL PRODUCTIONFROM WASTE COOKING OIL

(Aulia Syafitri, 2022, 41 Pages, 12 Tables, 9 Pictures, 4 Attachments)

Synthesis of biodiesel have been using more homogeneous catalysts suchasNaOH or KOH. The using of homogeneous catalysts NaOH or KOHin biodiesel production have some weaknesses, such as the formation of side products intheform of soap, and the complicated separation of the biodiesel product generatedwith the catalyst. To overcome these weaknesses, it began to be developed the useof heterogeneous catalysts (solid) to replace homogeneous catalyst. Homogeneouscatalyst can be replaced with more environmentally friendly heterogeneouscatalyst, stable at high temperatures, have bigger pores and relatively cheap prices. The creation of a heterogeneous catalyst can be done with utilization of rice huskand eggshell that produce the catalyst CaO/SiO2. CaO which is used comes fromchicken egg shell waste. The main component of the egg shell is CaCO3 canbeconverted into CaO through calcination process. Utilizatiion of rice huskwill produce silicate that can be used for various chemical processes, such as catalyst support. This study varied the ratio of CaO/SiO2 (5:3, 5:5, and 5:7) w/wandimpregnation time (22, 23, and 24) hours. The best catalyst is seen fromyieldandthe quality of biodiesel produced. The best yield result is 71,54%in ratio 5:7w/wwith impregnation time is 24 hours.

Keywords : Catalyst, Eggshell, Rice Husk, CaO/SiO2, Biodiesel